wifi-csi-presence-sensing

Technical documentation

Presence, motion and position sensing with ESP32 WiFi CSI

# Contents

# 1. Overview

This project detects whether someone is in a room, roughly where they are and how much they're moving, using only WiFi. Small ESP32-S3 boards listen to the signal that a normal router is already sending. No cameras, microphones or wearables are involved.

A person's body, which is mostly water, absorbs and reflects radio waves. When someone walks into a room, the way the router's signal bounces around changes, and the boards can measure that. They can't see anything. They only measure how the signal arriving from the router has changed.

What the boards report is Channel State Information (CSI). A WiFi channel is split into many narrow frequency bands called subcarriers, and for each one CSI gives the strength and phase the signal arrived with. The backend turns that into an occupied or empty decision, a rough position and a motion level, and the dashboard shows the result live.

It doesn't do pose estimation. The figure in the dashboard's 3D view is a marker placed at the estimated position, not a tracked body. The breathing and heart rate readouts aren't reliable on this hardware; section 8 explains why.

# 2. How the pieces fit together

| Stage | Part | Output |
| --- | --- | --- |
| Capture | ESP32-S3 boards running the RuView CSI firmware | CSI frames and summary packets, sent over UDP |
| Processing | backend/csi_bridge.py (Python, FastAPI) | Presence, position, motion level, smoothed vitals |
| Delivery | WebSocket at /ws/sensing | JSON sensing_update messages, several per second |
| Display | React and Three.js dashboard | Node status, motion level, vitals readout, 3D room view |

The boards only do what has to happen next to the radio. Everything that needs tuning, like presence thresholds, the position maths and smoothing, runs in Python on the PC, so it can be changed without reflashing anything.

# 3. Hardware

| Part | Notes |
| --- | --- |
| ESP32-S3, 8 MB flash | About $9. One board gives presence; two or three are needed for position. It has to be an S3: the firmware runs WiFi on one core and the signal processing on the other, and single-core chips like the ESP32-C3 can't run it. |
| 2.4 GHz WiFi router | The signal being measured. The ESP32 only captures CSI on 2.4 GHz. |
| PC | Runs the backend (HTTP and WebSocket on port 4000, UDP on 5005) and serves the dashboard. |

Placement matters. A person shows up most clearly when their body is between a board and the router.

# 4. Firmware

The firmware is RuView's ESP32 CSI node firmware, version 0.7.0, included without changes. Core 0 runs the WiFi driver and copies each received frame's CSI into a ring buffer. Core 1 takes frames off the buffer, runs the signal processing (including the firmware's own breathing, heart rate and presence estimates) and sends packets to the backend.

CSI only arrives when the router sends a frame, so the real rate on these boards was about 13 to 19 frames per second rather than the 100 Hz the firmware is designed for.

## Packets

| Magic | Size | Contents |
| --- | --- | --- |
| 0xC5110001 | 20-byte header + I/Q | Raw CSI: node ID at byte 4, subcarrier count at bytes 6-7, RSSI at byte 16, then one int8 I and Q pair per subcarrier. Amplitude is sqrt(I^2 + Q^2). |
| 0xC5110002 | 32 bytes | Vitals: flags at byte 5, breathing rate x100 at bytes 6-7, heart rate x10000 at bytes 8-11, RSSI at byte 12. |
| 0xC5110003 | 48 bytes | Eight float32 summary features starting at byte 16. |
| 0xC5110004 | 48 bytes | Vitals merged with a 60 GHz mmWave sensor. Only sent if one is attached. |

Getting these byte offsets exactly right mattered. An early version of the backend read the vitals from the wrong offsets and produced believable but meaningless numbers.

## The multi-node bug

The first firmware build used, RuView v0.4.3.1, only let data from one node through. Each board's node ID is stored in flash, but starting the WiFi driver overwrote it, so every board reported itself as node 1 and the backend merged them into a single sensor. A second or third board added nothing, and position estimation, which needs several boards, couldn't work.

RuView v0.7.0 reads the node ID right after loading its configuration and before WiFi starts (csi_collector_set_node_id(), called from main.c), and uses that copy from then on. Moving to v0.7.0 fixed the problem, and the prebuilt images in the repository are that version.

# 5. Backend

backend/csi_bridge.py receives the UDP packets on port 5005, decodes them by their magic number and keeps separate state for each node. It can also read from a RuView sensing server on port 3000 if one is running, but the direct UDP path is the one this project uses.

## Presence detection

Presence comes only from the raw CSI frames. For each node:

- Each frame's subcarrier amplitudes are divided by that frame's mean amplitude. This removes the effect of the ESP32's automatic gain control, which rescales whole frames for reasons unrelated to people.

- The last 60 normalized frames are kept, and the standard deviation of each subcarrier over that window is calculated.

- The 90th percentile of those values is the motion metric. A high percentile rather than an average lets the few subcarriers a person affects drive the result.

- The metric is compared to a baseline kept for that node. The baseline falls quickly and rises slowly, so it settles at the empty-room level. The node reports presence when the metric is more than twice the baseline and above 0.12 for four frames in a row.

The room counts as occupied if any node reports presence. The firmware's own presence flag is ignored: in testing it never turned on, even with someone standing in the room.

## Position

Each node's RSSI is converted to a distance with a log-distance path-loss model, distance = 10^((-40 - RSSI) / 30) metres, limited to between 0.3 and 12 m. With two nodes the backend finds where the two distance circles intersect and picks the point nearer the middle of the room. With three or more it takes an average of the node positions weighted by 1/distance. The node coordinates are set in NODE_POSITIONS at the top of csi_bridge.py.

## Smoothing

Position is smoothed with an exponential moving average that gives each new value a weight of 0.15. Breathing and heart rate get the same treatment with a weight of 0.25. A reading that's briefly out of range keeps the previous value instead of dropping to zero, and both are cleared when presence is lost.

## Delivery

Each update becomes a JSON sensing_update message (people and their positions, occupancy, motion level, vitals and the node list) broadcast over the /ws/sensing WebSocket. Updates triggered by CSI frames are limited to five a second.

# 6. Dashboard

The dashboard is a Vite and React app. The useRuView hook keeps the WebSocket open, reconnects if it drops and parses each message. Position goes through a one-dimensional Kalman filter on each axis. Vitals stay on screen for one second after they drop out, and a person stays for 0.8 seconds after the last occupied frame, so one missed frame doesn't make the display flicker.

Room3D.jsx draws the room with Three.js and puts the person marker at the centre of the zone they're in, because the position isn't more precise than that.

# 7. What didn't work

## Averaging the subcarriers

The first presence detector averaged the amplitude across all subcarriers in each frame and tracked how that average varied. An empty room and an occupied room gave the same reading, around 0.6 to 0.85 on its motion scale. Averaging hid the few subcarriers a person affects, and the variation that was left came mostly from automatic gain control.

## Trusting the firmware's vitals packets

A version that took presence from the firmware's vitals packets kept the room marked occupied indefinitely after everyone left, because the firmware kept sending plausible-looking breathing and heart rates.

## The pre-trained model

RuView links a pre-trained model on Hugging Face. Its model card describes ONNX files and a head that outputs breathing and heart rate, but the repository has neither. It contains a small encoder (8 inputs, 64 hidden units, 128 outputs) and one presence head. Run on real data, the presence head said occupied for every input, empty room included, because of a large bias term. It isn't used.

# 8. Limitations

- Breathing and heart rate aren't reliable. The firmware estimates them by filtering the CSI phase and counting zero crossings, and with nobody in the room it still reported breathing rates of 15 to 39 a minute and heart rates of 45 to 110 bpm. A heartbeat moves the chest about 0.2-0.5 mm, roughly a hundred times less than breathing, and breathing harmonics fall inside the heart-rate band.

- Only amplitude is used. The ESP32's CSI phase is corrupted by carrier frequency and timing offsets.

- Position is rough, about 1-2 m, and it jitters. A precise position needs angle of arrival, which needs CSI phase from several antennas, and each ESP32 has one.

- Placement makes a big difference. In testing, a badly placed node barely reacted to a person standing in the room.

- Nodes should stay on one WiFi channel. Channel hopping looks like movement and causes false presence.

- Start the backend with the room empty. Each node seeds its baseline from its first frames, so someone already in the room becomes part of the baseline.

- The room occasionally flips back to occupied a few seconds after it empties. The likely causes are channel hopping or something moving near a node, such as a fan.

# 9. What it can and can't do

| Capability | Status |
| --- | --- |
| Presence (occupied or empty) | Works |
| Motion level | Works |
| Someone entering or leaving | Works, since it follows from presence |
| Position | Rough: about 1-2 m, good enough to tell zones apart |
| Breathing rate | Experimental. Needs phase processing and a person sitting still |
| Heart rate | Not reliable with ESP32 CSI. Needs a mmWave radar |
| Precise position tracking | Needs hardware with several antennas |
| Telling people apart | Not possible with this setup |

From cheapest up, the things that would improve it are: locking every node to one channel, recovering the CSI phase in software, adding nodes, adding a 60 GHz mmWave radar for vitals (the firmware already has a packet type for it), and using a WiFi card that reports CSI from several antennas for angle of arrival. docs/RESEARCH.md goes into more detail.

# 10. Glossary

| Term | Meaning |
| --- | --- |
| CSI (Channel State Information) | How the WiFi signal changed between the router and the board, reported per subcarrier as amplitude and phase. |
| Subcarrier | One of the narrow frequency bands a WiFi channel is divided into. |
| I/Q | The two parts of each CSI sample. Amplitude is sqrt(I^2 + Q^2). |
| AGC (automatic gain control) | The chip adjusting its own gain. It changes every subcarrier's amplitude at once, which is why frames are normalized. |
| RSSI | Received signal strength: a single number for the whole frame. |
| EMA (exponential moving average) | A running average in which each new value gets a fixed weight. |
| Hysteresis | Needing several readings in a row to agree before the state changes. |
| Trilateration | Working out a position from distances to several known points. |